NAME 	____________________________________  	INDEX NO.		_______________

SCHOOL 	____________________________________	SIGNATURE 		_______________									DATE 			_______________

232/2

PHYSICS

PAPER 2

(THEORY)

TIME: 2 HRS

FORM 4

INSTRUCTIONS

- Write your name, school and your index number in the spaces provided.

- Sign and write the date of examination in the spaces provided above.

- This paper consists of two sections, Section A and B.  Answer ALL the questions in both sections in the spaces provided in this paper.

- ALL working must be clearly shown.

- KNEC mathematical tables and non-programmable silent electronic calculators may be used.

Note: Take acceleration due to gravity g = 10m/s2

FOR EXAMINER’S USE:

| SECTION | QUESTION | MAXIMUM SCORE | STUDENTS SCORE |
| --- | --- | --- | --- |
| A | 1-12 | 25 |  |
| B | 13 | 12 |  |
|  | 14 | 11 |  |
|  | 15 | 13 |  |
|  | 16 | 08 |  |
|  | 17 | 11 |  |
|  | TOTAL | 80 |  |

This paper consists of 13 printed pages

Candidates should check to ensure that all pages are printed as indicated and no questions are missing

SECTION A (25 MARKS)

Answer ALL the questions in this section in the spaces provided.

- State any one condition under which a pinhole camera may form an image on its screen which

has the same size as the object.									(1 mark)

______________________________________________________________________________________________________________________________________________________________________

- The source of sound  and two listeners are positioned close to a tall building as shown in the

figure below.

Source of Sound

Listener B

Listener A

- State the property of sound by which Listener A is able to hear the sound produced.		(1 mark)

___________________________________________________________________________________

- Listener B is moving directly towards Listener A and has a problem hearing the sound produced. Explain.												(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- Give two differences between a transformer and induction coil. 				(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- The figure below shows some region of part of the electromagnetic spectrum.

Red			Violet

| A | B | C |  | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |

- Name the region that is detected by a blackened bulb thermometer. 				(1 mark)

___________________________________________________________________________________

- State one use of the E-M wave in the region labelled B.						(1 mark)

______________________________________________________________________________________________________________________________________________________________________

- The figure below shows ultraviolet light striking a clean Zinc plate on a positively charged leaf electroscope.

U.V

Explain the following observations:

- The leaf does not fall.										(1 mark)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- When the same experiment is carried with a negatively charged electroscope, the leaf falls.	(1 mark)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- A girl observes her face in a concave mirror of a focal length 90cm. If the mirror is 70cm away,

state two characteristics of the image observed.							(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

3.0V, 0.5

The ammeter in the circuit in figure 3 has negligible internal resistance. The cell has an internal resistance of 0.5Ω and an electromotive force of 3.0V.

S

3.5

Figure 3

Determine the value of current the ammeter registers when switch S is closed.			(2 marks)

- An electric heater is rated 3 kW. Find the electrical energy in kWh(kilo watt- hours) consumed

by the heater when used on a 240V supply for 180 minutes. 					(2 marks)

- An echo sounder produces a pulse and an echo is received from the seabed after 0.4 seconds.

If the speed of sound in water is 1500m/s. Calculate the depth of the sea bed.			(2 marks)

- The set up in figure 4 below can be used in a laboratory for lifting and releasing a steel ball.

i) 	State the material which is suitable for use in the core.						(1 mark)

___________________________________________________________________________________

- If a slightly larger ball is to be lifted, it is necessary to make an electromagnet stronger.

Name two ways of increasing the strength of the magnet.					(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Air

The figure 5 below shows the path of a ray of light passing through a rectangular block of Perspex placed in air.

Ray of light

Perspex

Calculate the refractive index of Perspex.							(2 marks)

- The sketch shown below is a displacement-time graph of a wave traveling at 320ms-1

Find the wavelength of the wave.								(2 marks)

SECTION B (55 MARKS)

Answer ALL the questions

- a) 	The mains electricity is transmitted through high tension voltage (H.T).

State one danger of this transmission.							(1 mark)

______________________________________________________________________________________________________________________________________________________________________

Bulbs

b) 	The figure shows a section of a domestic wiring.

Fuse

L

N

i) 	Identify two serious mistakes in the circuit.							(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

ii) 	Explain why the circuits in domestic wiring should be connected in parallel with the main supply.														(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

c) 	Study the figure shown below

R1

R2

B2

B1

S

Q

P

Brushes

R

B

A

Magnet

i) 	State the name of the machine shown in figure above.					(1 mark)

___________________________________________________________________________________

ii) 	What are the names of the parts labelled A and B?						(2 marks)

A		___________________________________________________________________________

B		___________________________________________________________________________

- What would be the effect of doubling the number of turns of the coil if the speed of rotation remained unchanged?										(1 mark)

______________________________________________________________________________________________________________________________________________________________________

d) 	The voltage-time signals obtained is as shown in the diagram below in graph A

i) 	Find the frequency of the alternating E.m.f shown by graph A.				(2 marks)

ii) 	What alterations have been made to the machine in c) above to produce the emf represented

by graph B?											(1 mark)

Target

The figure below shows the features of an X- ray tube

+

E.H.T

X-rays

Q

P

Cooling fins

Oil

Filament

A

B

a) 	Name the parts labelled A and B.								(1 mark)

A		___________________________________________________________________________

B		___________________________________________________________________________

b)	Explain how a change in the potential across PQ changes the intensity of the X-rays produced

in the tube.											(2 marks)

___________________________________________________________________________________

___________________________________________________________________________________

___________________________________________________________________________________

___________________________________________________________________________________

c) 	What property of lead makes it suitable for use as shielding material?			(1 mark)

___________________________________________________________________________________

___________________________________________________________________________________

- In an X-ray tube operating at 100kV, the tube current is 20mA. Determine the number of

electrons hitting the target every second.							(2 marks)

(Charge of an electron = 1.6 × 1019C)

- The figure below shows a cell. Use it to answer questions that follow:

Anode

- Name the type of cell.										(1 mark)

___________________________________________________________________________________

ii) 	Label the cathode.										(1 mark)

iii)	The maximum kinetic energy of the photo-electrons emitted from a metal surface is

9.95 x 10-20J. If the threshold frequency of light required to cause photo-electric emission

with this metal is 5 x 1014 Hz. Calculate the frequency of the incident radiation.

(Plank’s constant = 6.63 x 10-34 JS) 								(3 marks)

- a) 	i) 	Explain why carbon -14 (614 C) is radioactive while carbon 612 C is not.			(1 mark)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

ii)	A radioactive isotope showed a count rate of 82 counts per second initially. After a time of 210 		seconds, the count rate dropped by 19 counts per second. The average background count 			remained constant at 10 counts per second. What is the half-life of the material?	(2 marks)

Perpex lid

b)	The figure below shows features of a diffusion cloud chamber used for detecting radiations from a 	radioactive source.

Black velvet floor

Light source

Felt soaked in alcohol

Solid carbon dioxide (dry ice)

Source

Foam

Explain how the chamber works when a radioactive particle is introduced at the source.	(2 marks)

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

c) 	i)	What is the purpose of solid carbon (IV) oxide.						(1 mark)

__________________________________________________________________________________________________________________________________________________________________

ii) 	State one advantage of the cloud chamber over a G.M  tube as a detector of radioactive 	radiations.											(1 mark)

__________________________________________________________________________________________________________________________________________________________________

d) 	P-type and n-type semiconductors are made from a pure semiconductor by a process known as “doping”.

i) 	State what is meant by doping.								(1 mark)

__________________________________________________________________________________________________________________________________________________________________

ii) 	Explain how the doping produces an n-type semi-conductor.				(2 marks)

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

e) 	In the circuit in the figure below, when the switch S is closed, the voltmeter shows a reading.

S

R

D

V

Cell

When the cell terminals are reversed and the switch is closed, the voltmeter reading is zero. Explain these observations.										(2 marks)

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

- a) 	The figure below shows the features of a cathode ray oscilloscope;-

Filament

X-plates

Screen

Vacuum

Vacuum

B

A

Cathode

Anodes

- Name the parts A and B. State the role played by each of the parts A and B.		(2 marks)

A		___________________________________________________________________________

___________________________________________________________________________

B		___________________________________________________________________________

___________________________________________________________________________

ii)	Explain how electrons are produced. 								(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

iii) State the factors considered when choosing the material for the cathode. 			(1 mark)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

b) 	The figure below shows the trace on the screen of an a.c signal connected to the y-plates

of a C.R.O with time base on. Given that the time base control is 100ms/div and the y-gain

is at 120V/division.

Determine:

i) 	The frequency of the a.c signal.								(2 marks)

ii) The peak voltage of the input signal.			 					(2 marks)

- a) The figure below shows straight wave fronts incident on convex reflector.

Complete the diagram to show the reflected wave fronts.					(1 mark)

b)	Narrow slits cannot be used to demonstrate interference of sound waves. Explain.		(1 mark)

______________________________________________________________________________________________________________________________________________________________________

Wave fronts

c) The figure below shows sound waves emitted by a drum struck.

Explain why the wave fronts are directed to the ground.					(2 marks)

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

d) 	Figure below shows an experimental set up consisting of a mounted lens, I, A screens,

a metre rule and a candle

Flame

Screen (S)

L

i)	Describe how the set up may be used to determine the focal length f, of the lens.		(3 marks)

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

ii) 	State why the set up would not work if the lens were replaced with a diverging lens.	(1 mark)

______________________________________________________________________________________________________________________________________________________________________

e) 	The graph figure below shows the relationship between 1u and 1v for a converging lens where u and v 	are the object and image distances respectively.

1u×10-2(cm-1)

From the graph, determine the focal length, f of the lens.					(3 marks)