Nyaraya Cluster Examination

Kenya Certificate of Secondary Education

# 2023 Form Four Evaluation Programme

448/1	ELECTRICITY               Paper 1

JULY/AUGUST 2023

Name……………………………………………………..…Index no…………………                                                                                        Candidate’s signature……………

ELECTRICITY

PAPER 1

(THEORY)

2 ½ hour

Kenya certificate of secondary education

ELECTRICITY

PAPER 1

(THEORY)

Instructions to candidates

- This paper consists of two sections: A and B

- Answer all the questions in section A and any four questions from section B in the spaces provided

- All dimensions are in millimeters unless otherwise stated

SECTION A (48 marks)

1 a) State four categories of institutions that train electrical technicians in Kenya 		(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b) List four key components of a business plan 					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

2a)  State how each of the following electrical wates materials should be disposed.		(1mks)

i) 	Lead

……………………………………………………………………………………………………………………………………………………………………………………

ii)	Florescent tube

……………………………………………………………………………………………………………………………………………………………………………………

b). State how each of the following type of fire extinguisher is suitably applied		(3mks)

- Foam

……………………………………………………………………………………………………………………………………………………………………………………

- Water

……………………………………………………………………………………………………………………………………………………………………………………

- Dry powder

……………………………………………………………………………………………………………………………………………………………………………………

3 A one –watt resistor has the colour code blue, grey, and brown. Determine

- The value of the resistor 					(1mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- The maximum value of the current that can flow through it without exceeding its power rating      					(4mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

4a) State lenz’s law of electromagnetic induction                                 									(1mk)

…………………………………………………………………………………………………………………………………………………………………………........................................................

b). State two characteristics of magnetic lines of force                 				 	(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………5 a) Explain the meaning of sensitivity as used in meter movement 				(1mk)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). With the aid of a diagram, show how the linearity of a meter is determined 		(4mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

6 a) With the aid of a diagram, describe armature reaction in d.c. generator  		(4mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). Outline two methods of reducing armature reaction. 					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

7a) Distinguish between intrinsic and extrinsic semi –conductors and give one example of each. 													(3mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). List four uses of an ohmmeter in trouble shooting electric circuits  			(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

8 a) Name four conductor materials used in electric circuits 					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). State two advantages of MIMS over PVC cables   											(2mks) ………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………….......................................................

9. Figure 1 shows a series –parallel circuit connected across a 240V supply

Calculate the:

- Total circuit current					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Voltage drop across 					(4mks)

- R3

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- R4

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………..

10 a) Name four marking out tools used in metal fabrication   					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). Figure 2 shows the orthographic views of a bracket drawn in first angle projection

Taking N as the lowest point,make a free hand isometric sketch of the bracket 	(4mks)

SECTION B (52 marks)

11.a) Convert 									(4mks)

i) 4110 to binary

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

ii) 11011012 to decimal

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). Sketch the symbol for each of the following logic gates					(3mks)

i) AND

- OR

iii). NAND

12. a) State three advantages of toroidal type transformer over shell type transformer 	(3mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). i) Outline three assumptions made in order to consider a transformer as an ideal machine.

(3mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

ii). A 500/500V,10KVA ideal single –phase transformer has 40 turns on the secondary. Calculate

i) Primary turns 					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

ii) Primary full load current					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

iii) Secondary full load current  					(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

13. a) State the phase relationship between current and voltage in circuits that are purely. (2mks)

i) Resistive

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

ii). Inductive

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). Figure 3 shows an RLC circuit

Calculate the 				        (11mks)

- Inductive reactance

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Capacitive reactance

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………….............................................................................

- Circuit impedance

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Circuit current

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Power dissipated in the circuit

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………..

14 a) State					(4mks)

- Two IEE requirements regarding bell transformers

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Two advantages of MCB over cartridge fuses

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). Outline the procedure of carrying out an insulation resistance test on a new domestic insulation  					(9mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

15. a) With the aid of a diagram, explain how the right hand grip rule is used to determine the direction of the magnetic field around a current carrying conductor 			(3mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

b). With the aid of a labelled diagram, explain the principle of operation of a trembler bell.

(10mks)