Nyaraya Cluster Examination

Kenya Certificate of Secondary Education

# 2023 Form Four Evaluation Programme

447/1	POWER MECHANICS	Paper 1

JULY/AUGUST 2023

447/1

POWER MECHANICS

Paper 1 (THEORY)

July/August

TIME -21/2

NAME …………………………………………… Index Number …………………

Candidate’s signature …………………… Date ……………………………

Instructions to candidates

- Write your name and index number on the spaces provided above.

- Sign and write the date of examination in the spaces provided above.

- Candidates should have the following for this examination:

- Drawing instruments

- Scientific calculator

- Drawing paper size A4

- This paper contain two sections A and B

- Answer all questions in section A  in the spaces provided.

- Answer question 11 on A4 paper and any other three questions from section B in the spaces provided.

- All dimensions are in millimetres unless otherwise stated.

For examiner’s use only

| Section | question | Maximum score | Candidates score |
| --- | --- | --- | --- |
| A | 1-10 | 40 |  |
| B | 11 | 15 |  |
|  |  | 15 |  |
|  |  | 15 |  |
|  |  | 15 |  |

SECTION A (40 marks)

Answer all questions

- (a) Define a business plan.                                                                 		(2mks)

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(b) List down four factors that must be considered before setting up a ‘’jua kali’’ garage.                                                                             				(2mks)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………….…………………………………………………..

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………...

- (a) An engine with a good compression starts but does not run. Name any four engine systems that could be faulty.                                  			  (2mks)

.................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

(b) State two advantages of independent front suspension over rigid beam suspension

(2mks)

.................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

- (a) State two reasons for having a first aid box in a vehicle.          		(2mks)

..................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

(b) Use sketches to differentiate between slotted hexagonal nut and a castellated nut

(2mks)

- (a) State three functional characteristics of a road wheel.            		(3mks)

..................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

(b) State two advantages of riveting over soldering.               			(1mk)

.................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

- (a) Name four main electrical circuits in a motor vehicle.                  		 (2mks)

..................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

(b) State two types of bearing loads and for each state an area of application in motor vehicle.                                                 							 (2mks)

......................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

6. State the function of each of the following components: 				(4mks)

(i). Ball joint …………………………………………………………………………………………….............................................................................................................................

(ii). Damper …………………………………………………………………………………………………………………………………………………………………………………………

(iii). Wrist …………………………………………………………………………………………………………………………………………………………………………………………

(iv). Brake drum

……………………………………………………………………………………………..............................................................................................................................

7. (a) State the purpose of each of the following tools:					 (2mks)

(i). Tachometer ……………………………………………………………………………………………..............................................................................................................................

(ii).Telescopic gauge ……………………………………………………………………………………………..............................................................................................................................

(b) List four cleaning tools used during soldering.                 				 (2mks)

……………………………………………………………………………………………..............................................................................................................................

………………………………………………………………………………………..............................................................................................................................

8. (a) Explain the difference between flushing and bleeding as used in braking system

(2mks)

……………………………………………………………………………………………..............................................................................................................................……………………………………………………………………………………………..............................................................................................................................

(b) State four components operated by a multi-cylinder engine cam-shaft.    (2mks)

……………………………………………………………………………………………..............................................................................................................................

……………………………………………………………………………………………..............................................................................................................................

9. (a) State four causes of vehicle swaying on turn                                  		 (2mks)

……………………………………………………………………………………………..............................................................................................................................……………………………………………………………………………………………..............................................................................................................

(b) Explain the meaning of each of the following fabrication process.

(i)  Forming                                                                                           		 (1mk)               ………………………………………………………………………………………………………………………………………………………………………………………….

(ii)  Finishing                                                                                            		(1mk)                 ……………………………………………………………………………………………..............................................................................................................................

10. (a) State the energy conversion that take place in each of the vehicle components:

Horn                                                                                              			 (1mk) ……………………………………………………………………………………………..............................................................................................................................

Alternator                                                                                     			(1mk) …………………………………………………………………………………………….............................................................................................................................

(b) Sketch the symbols for each of the following electrical devices.  		(2mks)

- Lamp

- Zener diode

- Capacitor

- Battery

SECTION B (60 marks)

Answer question 11 and any three questions from this section in the spaces provided. Candidates are advised not to spend more than 25 minutes on question 11.

11.  A shaft support block is shown in isometric in the figure below. Draw in third angle orthographic projection:

- The plan view.

- The sectional front view in the direction A-A.

12. (a) State four functions of oil in an engine.                                                     (4mks)  ………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(b) Using labelled sketch, explain the operation of pressure relieve valve.      (7mks)

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(c) Describe the splash feed lubrication system as used in single cylinder engines.

(4mks)

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

13. (a) State one function of each of the following charging system components.(3mks)

- Diode  …………………………………………………………………………………………………………………………………………………………………………………………

- Cut-out relay   …………………………………………………………………………………………………………………………………………………………………………………………

- Pole shoes.   ………………………………………………………………………………………………………………………………………………………………………………………… (b) List four differences between D.C generator and an alternator.                  (4mks)

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(c) Describe the operation of an alternator.                                                         (8mks)

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

14. (a) List four constant velocity universal joints.                                                  (2mks)

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(b) Name four gears used in the final drive.                                                             (2mks)

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(c) (i) Sketch and label all the parts of a final drive.                                                (5mks)

(ii)Explain the differential principle during cornering.                                       (6mks)

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

15. (a) State three causes of each of the following in oxy-acetylene welding.         (3mks)

- Backfire   ………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Flash back  …………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(b) Explain four effects of oxy-acetylene flame on steel.                 		(8mks)

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

(c) State four advantages of rightward method over leftward welding method.   (4mks)

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………